Plus One Flyers

Cessna 182 T Nav III Checkout

INSTRUCTOR:			_______________________

STUDENT:			_______________________

DATE COMPLETED		_______________________

OBJECTIVE

With the use of the Pilot’s Information Manual, the pilot will be able to identify information relating to the safe operation of the Cessna 182 T.  A reference number that indicates the page in the POH where information on the question may be found follows each question.

GENERAL

- What are the fuel selector positions and how should they be used for enroute, landing and parking? (1-6)

- If a vacuum failure occurs, will the autopilot still function?  Why?

- If an alternator failure occurs, how long will the standby battery provide power?  What procedure should be used? (3-16) & (3-18)

- How do you adjust the PFD and MFD for day/night flying? (7-58)

- What are the 5 ways the autopilot can be disengaged? (Supplement 3-9)

6.   What are the cold, hot, and flooded start procedures? (4-27)

AIRCRAFT LIMITATIONS

1. What is the Never Exceed Speed (Vne)? (2-4)

2. What is the Maneuvering Speed (Va) at 3,100 lbs? (2-4)

3. What is the Maximum Flap Extended Speed (Vfe)? (2-4)

10 Degrees

20 Degrees

30 Degrees

4. What is the Maximum Takeoff Weight? (2-8)

5. What is the Maximum Weight in the following baggage compartments? (2-8)

A

B

C

6. What is the Maximum Usable fuel quantity? (2-14)

EMERGENCY PROCEDURES

1. What is the airspeed for maximum glide at 3,100 lbs.? (3-5)

2. What is the procedure for engine failure immediately after takeoff? (3-6)

NORMAL PROCEDURES

- What is the 0 degree flap Normal Climb speed after a normal takeoff? (4-9)

- What is the enroute climb power setting? (4-20)

- What is the Best Power leaning procedure? (4-35)

- How can this be accomplished using the lean assist function? (4-37)

- What is the flaps down final approach speed for a normal landing? (4-23)

- What is the starter motor cycle limit and cool down period? 4-28)

PERFORMANCE

- What is the stall speed (KIAS) with the most forward C.G., 3100 lbs, flaps up and a 45 degree bank? (5-13)

- What is the takeoff distance to clear a 50 ft obstacle using maximum performance procedures, at gross weight, 6,000 ft elevation, and 30 degrees C? (5-15)

- What is the expected fuel consumption at a cruise altitude of 10,000 ft with 69% power set at 2,300 rpm and 20 degrees above standard temperature? (5-29)

- What is the expected maximum range with 87 gallons useable fuel and a 45 min reserve at 75% power, 10,000 ft, zero wind, and standard temperature? (5-33)

- What is the landing distance over a 50 ft obstacle using maximum performance procedures, at 2,950 lbs, 6,000 ft elevation, and 30 degrees C? (5-36)

WEIGHT AND BALANCE

- Using the sample loading chart on page 6-11 as empty weight, what is the takeoff weight and CG with full tanks, a pilot weight of 200 lbs, a front passenger weight of 130 lbs, rear passenger weights of 180 lbs, and 130 lbs of baggage in baggage area C? (6-17).

- Is the weight and CG within limits? (6-17)

G1000 Avionics Ground Session(s)

- PFD Panel Information

- Altitude, A/S, VSI Tapes

- V Speeds on PFD

- Slip/Skid Indicator

- Setting barometric pressure

- Glide slope indicator

- Altitude bug

- Current heading

- Selected heading

- Heading bug

- Centering heading

- Selected course

- Centering HSI

- Standard rate turn

- Current track bug

- Timer

- Map Inset (Range/Declutter)

- TAS

- OAT

- Wind Data

- METARS/TAFS

- CDI nav source function

- Bearing (RMI) pointers

- Transponder operation

- Range/Joystick operation

- CLR button to return to NAV

- Range Knob

- FMS/GPS/Menu buttons

- Setting Com frequencies

- Setting Nav frequencies (ID)

- Volume for Nav & Com

- MFD Panel Information

- Manifold pressure indicator

- RPM indicator

- Engine indicating system

- Electrical indicating system

- Fuel indicating system

- Gals remaining/fuel reset procedure

- Vacuum system

- Engine lean assist

3)	G1000 Pages

- Map Pages

- Waypoint Pages

- Auxiliary Pages

- Nearest Pages

- Flight Plan Pages

- Procedure Pages

4)  Audio Panel Operation

- Intercom volume

- Pilot/co-pilot isolation

- Playback feature

- Auto Pilot Operation

- Autopilot test procedure

- ROL, HDG, NAV, APR, ALT, UP, DN, BARO

- 406 ELT Operation

- Master switch on

- Press switch until sound is heard

- Release switch

- Look for 1 flash

If more than one flash or no flash occurs, 406 is malfunctioning

GPS linkage location accuracy 1/2

mile

7)	Connecting Battery Charger

- Need screwdriver to open door

- Charger on “Flooded” setting

- Turn on only Bat side of Master

- Return everything to office

Flight Session(s)

   Pre-Flight Procedures

1)	Preflight per the POH

2)	Automatic alternate air

3)	Static ports/pitot source

- Antenna locations:

- Marker beacon – tail cone

- Transponder – bottom of cabin

- Stormscope – bottom of cabin

- 2 TAS antennas

  - 1 top of cabin (directional)

  - 1 tail cone (omni)

- Com

- GPS

- Fuel dipstick & useable fuel

- Fuel sample locations (13)

- Oil level & filler spout (4-9 qts)

- Cowl flaps open

- Strut height/tire pressure

Nose 49 PSI Mains 42 PSI

Strut pressure 55-60 PSI

- Proper glass care inside and out:

- AROW documents

- Review light dimmers

- Interior night lighting

- Location of back-up instruments

- TC & autopilot loss of PFD operation

- Standby battery switch use

   Starting and Run Up

1) Priming

2) Leaning for taxi

3) Cold and hot start procedures

4) Run up checks and procedures

 Taxi

- Taxi checks AI, ASI, DG, MC

- Range knob for taxi

 Takeoff

1) Normal, Short and Soft Field

 Climb and cruise

- Proper power settings

- Leaning for fuel flow (lean assist)

- Cowl flaps

  Maneuvers at Altitude

- Slow flight

- Dep, app, & accelerated  stalls

- Steep turns

- Simulated go-around with full flaps

  GPS and Autopilot use for a VFR pilot

- Basic HDG and ALT mode on AP

- AP induced stall in ALT mode or VS mode

- AP disconnects

- GPS “direct to” waypoint

- Basic G1000 use & procedures

- Loading VFR flight plans

  Use of GPS and Auto Pilot for the IFR pilot (in addition to the above)

- Loading IFR flight plans in G1000

- Flying coupled approaches with AP

 Emergency Procedures

- Emergency landing procedures

- Use of nearest airport

- Vacuum system failure – only affects backup AI

- Electrical failure, reversionary mode

  Pattern Work and Landing

- Proper power settings and airspeeds for downwind base and final

(See Configuration Page)

- Full flap and no flap landings

- Short & soft field landings

- Proper landing procedures: Land on mains, feet off brakes, use aerodynamic braking

- Pre-landing and post-landing checklists

  Shutdown and Mooring

- Use shut down checklist

- Standby battery switch is off

- Fuel selector on L or R

- Cowl flaps open

- Don’t slam doors with handle down

 Resources Available

- G1000 manual

- G1000 DVD

- KAP 140 autopilot manual

- Battery charger for G1000 learning

| Power Settings for Cessna 182T |  |  |  |
| --- | --- | --- | --- |
|  |  | Power/RPM | Result (Kts) |
| Best Angle of Climb VX |  | Full 2400 RPM | 65 |
| Best Rate of Climb |  | Full 2400 RPM | 80 |
| Cruise Climb |  | 23” 2400 RPM | 90 |
| Normal Cruise |  | 23” 2300 RPM | 145 |
| Approach Level (Flaps 10º) |  | 16” 2300 RPM | 90 |
| Precision Descent (Flaps 10º 500 FPM) |  | 12” 2300 RPM | 90 |
| Non Precision Descent (10º Flaps 800 FPM) |  | 10” 2300 RPM | 90 |
| Slow Flight (Flaps 30º) |  | 18” 2300 RPM | 60 |
| Downwind (No Flaps) |  | 16” 2400 RPM | 90 |
| Numbers (Flaps 10º) |  | 12” 2400 RPM | 90 |
| Base (Flaps 20º) |  | 12” 2400 RPM | 80 |
| Final (Flaps 30º) |  | 12” or Required | 70 |
| Short Field (Same as above except power at 13”) |  | 13” 2400 RPM | 60 |
| Airspeeds for Various Configurations |  |  |  |
| Best Glide | 76 |  |  |
| VA | 91-110 |  |  |
| VX | 65 |  |  |
| VY | 80 |  |  |
| VSO | 41 |  |  |
| VS1 | 51 |  |  |
| VR | 65 (Short Field 58) |  |  |
| VFE | <140 for 10º, <120 for 20º, <100 for 30º |  |  |
| VNO | 140 |  |  |
| VNE | 175 |  |  |